IV7 (IV7215) KV170 Integrated Power System Product Specification

IV7 (IV7215) KV170-Recommended for fixed-wing UAVs with a takeoff weight of 20~25kg

- 1.Parameters

| System Parameters | Model | IV7 (IV7215) KV170 |
| --- | --- | --- |
|  | Configuration | IV7 (IV7215) Motor+14S FOC IV150A ESC |
|  | Recommended Battery | 12~14S (LiPo) |
|  | Max Thrust (kg) | 14.7 |
|  | Recommended Takeoff Weight (kg) | 20~25 |
|  | Total Weight with Wires (g) | 1100±2% |
|  | Protection Rating | IP35 |
|  | Operating Temperature (℃) | -30~50 |
|  | Wire Length (mm) | 1800±5 |
|  | Wire Specifications | Power Wire: Silicon Wire-Red/Black-10AWG-1800mm Signal Wire: Shield Wire-Black-OD4.2-5C-1200mm Connector: -JP-3P*2-Black/White/Green/ Yellow/Gray |
| Motor | KV (RPM/V) | 170 |
| ESC Parameters | Throttle Range (μs) | 1040~1940 (Fixed) |
|  | Protocol | DRONE CAN/UAV CAN |
|  | Control Method | PWM/CAN |
|  | Max Voltage (V) | 60 |
|  | Max Continuous Current (A) | 80 (Open Environment, ≤60℃) |
|  | Peak Current (A) | 150 (Open Environment, ≤60℃) |
| Propellers | Model | 22*10 |
|  | Length (mm) | 558.8 |

| IV7 (IV7215) KV170+14S FOC IV150A+22*10 |  |  |  |  | Ambient Temp: |  | 25℃ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage (V) | Throttle | Current (A) | Thrust (gf) | Torque (N·m) | Speed (RPM) | Power (W) |  | Efficiency (gf/W) |
| 48 | 30% | 2.13 | 1005 | 0.34 | 2151 | 103 |  | 9.78 |
|  | 35% | 3.18 | 1439 | 0.46 | 2525 | 154 |  | 9.37 |
|  | 40% | 4.59 | 1978 | 0.61 | 2897 | 221 |  | 8.94 |
|  | 45% | 6.38 | 2562 | 0.77 | 3269 | 308 |  | 8.33 |
|  | 50% | 8.64 | 3248 | 0.98 | 3637 | 416 |  | 7.80 |
|  | 55% | 11.34 | 4015 | 1.18 | 4010 | 547 |  | 7.34 |
|  | 60% | 14.89 | 4867 | 1.41 | 4376 | 718 |  | 6.78 |
|  | 65% | 18.98 | 5809 | 1.67 | 4744 | 915 |  | 6.35 |
|  | 70% | 23.82 | 6833 | 1.97 | 5108 | 1148 |  | 5.95 |
|  | 75% | 29.41 | 7899 | 2.26 | 5474 | 1417 |  | 5.58 |
|  | 80% | 35.86 | 8957 | 2.56 | 5840 | 1727 |  | 5.19 |
|  | 85% | 43.58 | 10185 | 2.89 | 6215 | 2098 |  | 4.86 |
|  | 90% | 53.52 | 11645 | 3.30 | 6605 | 2575 |  | 4.52 |
|  | 95% | 74.03 | 13984 | 3.67 | 7160 | 3559 |  | 3.93 |
|  | 100% | 79.02 | 14667 | 4.16 | 7343 | 3795 |  | 3.87 |

- 2.Test Data

- 3.Outline Dimensions (mm)