IV7 (IV7215) KV200 Integrated Power System Product Specification

IV7 (IV7215) KV200-Recommended for fixed-wing UAVs with a takeoff weight of 20~25kg

- 1.Parameters

| System Parameters | Model | IV7 (IV7215) KV200 |
| --- | --- | --- |
|  | Configuration | IV7 (IV7215) Motor+14S FOC IV150A ESC |
|  | Recommended Battery | 12S (LiPo) |
|  | Max Thrust (kg) | 14 |
|  | Recommended Takeoff Weight (kg) | 20~25 |
|  | Total Weight with Wires (g) | 1100±2% |
|  | Protection Rating | IP35 |
|  | Operating Temperature (℃) | -30~50 |
|  | Wire Length (mm) | 1800±5 |
|  | Wire Specifications | Power Wire: Silicon Wire-Red/Black-10AWG-1800mm Signal Wire: Shield Wire-Black-OD4.2-5C-1200mm Connector: -JP-3P*2-Black/White/Green/ Yellow/Gray |
| Motor | KV (RPM/V) | 200 |
| ESC Parameters | Throttle Range (μs) | 1040~1940 (Fixed) |
|  | Protocol | DRONE CAN/UAV CAN |
|  | Control Method | PWM/CAN |
|  | Max Voltage (V) | 60 |
|  | Max Continuous Current (A) | 80 (Open Environment, ≤60℃) |
|  | Peak Current (A) | 150 (Open Environment, ≤60℃) |
| Propellers | Model | 20*10 |
|  | Length (mm) | 508 |

- 2.Test Data

| IV7 (IV7215) KV200+14S FOC IV150A+20*10 |  |  |  |  | Ambient Temp： |  | 25℃ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage (V) | Throttle | Current (A) | Thrust (gf) | Torque (N·m) | Speed (RPM) | Power (W) |  | Efficiency (gf/W) |
| 48 | 30% | 2.69 | 1087 | 0.36 | 2482 | 130 |  | 8.35 |
|  | 35% | 4.01 | 1565 | 0.48 | 2917 | 194 |  | 8.06 |
|  | 40% | 5.78 | 2159 | 0.64 | 3358 | 280 |  | 7.71 |
|  | 45% | 8.07 | 2798 | 0.81 | 3799 | 391 |  | 7.16 |
|  | 50% | 10.88 | 3496 | 1.00 | 4236 | 527 |  | 6.64 |
|  | 55% | 14.50 | 4318 | 1.25 | 4672 | 702 |  | 6.15 |
|  | 60% | 18.72 | 5200 | 1.50 | 5107 | 906 |  | 5.74 |
|  | 65% | 23.86 | 6197 | 1.74 | 5536 | 1154 |  | 5.37 |
|  | 70% | 29.99 | 7250 | 2.02 | 5967 | 1451 |  | 5.00 |
|  | 75% | 37.31 | 8439 | 2.33 | 6396 | 1804 |  | 4.68 |
|  | 80% | 46.11 | 9692 | 2.79 | 6833 | 2230 |  | 4.35 |
|  | 85% | 56.96 | 11082 | 3.20 | 7271 | 2752 |  | 4.03 |
|  | 90% | 70.47 | 12610 | 3.62 | 7734 | 3403 |  | 3.71 |
|  | 95% | 87.31 | 14237 | 4.11 | 8178 | 4211 |  | 3.38 |
|  | 100% | 87.33 | 14224 | 4.10 | 8175 | 4215 |  | 3.38 |

- 3.Outline Dimensions (mm)