IV9 (IV7224) KV160 Integrated Power System Product Specification

IV9 (IV7224) KV160-Recommended for fixed-wing UAVs with a takeoff weight of 28~32kg

- 1.Parameters

| System Parameters | Model | IV9 (IV7224) KV160 |
| --- | --- | --- |
|  | Configuration | IV9 (IV7224) Motor+14S FOC IV150A ESC |
|  | Recommended Battery | 12~14S (LiPo) |
|  | Max Thrust (kg) | 18 |
|  | Recommended Takeoff Weight (kg) | 28~32 |
|  | Total Weight with Wires (g) | 1300±2% |
|  | Protection Rating | IP35 |
|  | Operating Temperature (℃) | -30~50 |
|  | Wire Length (mm) | 1800±5 |
|  | Wire Specifications | Power Wire: Silicon Wire-Red/Black-10AWG-1800mm Signal Wire: Shield Wire-Black-OD4.2-5C-1200mm Connector: -JP-3P*2-Black/White/Green/ Yellow/Gray |
| Motor | KV (RPM/V) | 160 |
| ESC Parameters | Throttle Range (μs) | 1040~1940 (Fixed) |
|  | Protocol | DRONE CAN/UAV CAN |
|  | Control Method | PWM/CAN |
|  | Max Voltage (V) | 60 |
|  | Max Continuous Current (A) | 80 (Open Environment, ≤60℃) |
|  | Peak Current (A) | 150 (Open Environment, ≤60℃) |
| Propellers | Model | 24*12 |
|  | Length (mm) | 609.6 |

- 2.Test Data

| IV9 (IV7224) KV160+14S FOC IV150A+24*12 |  |  |  |  | Ambient Temp: |  | 25℃ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage (V) | Throttle | Current (A) | Thrust (gf) | Torque (N·m) | Speed (RPM) | Power (W) |  | Efficiency (gf/W) |
| 48 | 30% | 3.16 | 1579 | 0.55 | 2007 | 152 |  | 10.38 |
|  | 35% | 4.77 | 2197 | 0.75 | 2359 | 229 |  | 9.58 |
|  | 40% | 6.87 | 2934 | 0.97 | 2710 | 331 |  | 8.87 |
|  | 45% | 9.59 | 3778 | 1.24 | 3061 | 462 |  | 8.19 |
|  | 50% | 12.99 | 4717 | 1.52 | 3410 | 625 |  | 7.55 |
|  | 55% | 17.22 | 5790 | 1.87 | 3758 | 828 |  | 6.99 |
|  | 60% | 22.27 | 6960 | 2.21 | 4105 | 1071 |  | 6.50 |
|  | 65% | 28.39 | 8224 | 2.59 | 4451 | 1364 |  | 6.03 |
|  | 70% | 35.24 | 9620 | 3.07 | 4795 | 1694 |  | 5.68 |
|  | 75% | 44.34 | 11114 | 3.54 | 5148 | 2130 |  | 5.22 |
|  | 80% | 54.82 | 12787 | 4.10 | 5502 | 2632 |  | 4.86 |
|  | 85% | 68.17 | 14683 | 4.69 | 5872 | 3272 |  | 4.49 |
|  | 90% | 86.99 | 17035 | 5.43 | 6298 | 4170 |  | 4.09 |
|  | 95% | 98.84 | 18362 | 5.86 | 6510 | 4738 |  | 3.88 |
|  | 100% | 98.82 | 18341 | 5.86 | 6509 | 4736 |  | 3.87 |

- 3.Outline Dimensions (mm)