IV9 (IV7224) KV190 Integrated Power System Product Specification

IV9 (IV7224) KV190-Recommended for fixed-wing UAVs with a takeoff weight of 28~32kg

- 1.Parameters

| System Parameters | Model | IV9 (IV7224) KV190 |
| --- | --- | --- |
|  | Configuration | IV9 (IV7224) Motor+14S FOC IV150A ESC |
|  | Recommended Battery | 12S (LiPo) |
|  | Max Thrust (kg) | 17.5 |
|  | Recommended Takeoff Weight (kg) | 28~32 |
|  | Total Weight with Wires (g) | 1300±2% |
|  | Protection Rating | IP35 |
|  | Operating Temperature (℃) | -30~50 |
|  | Wire Length (mm) | 1800±5 |
|  | Wire Specifications | Power Wire: Silicon Wire-Red/Black-10AWG-1800mm Signal Wire: Shield Wire-Black-OD4.2-5C-1200mm Connector: -JP-3P*2-Black/White/Green/ Yellow/Gray |
| Motor | KV (RPM/V) | 190 |
| ESC Parameters | Throttle Range (μs) | 1040~1940 (Fixed) |
|  | Protocol | DRONE CAN/UAV CAN |
|  | Control Method | PWM/CAN |
|  | Max Voltage (V) | 60 |
|  | Max Continuous Current (A) | 80 (Open Environment, ≤60℃) |
|  | Peak Current (A) | 150 (Open Environment, ≤60℃) |
| Propellers | Model | 22*10 |
|  | Length (mm) | 558.8 |

- 2.Test Data

| IV9 (IV7224) KV190+14S FOC IV150A+22*10 |  |  |  |  | Ambient Temp: |  | 25℃ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voltage (V) | Throttle | Current (A) | Thrust (gf) | Torque (N·m) | Speed (RPM) | Power (W) |  | Efficiency (gf/W) |
| 48 | 30% | 3.85 | 1498 | 0.47 | 2521 | 185 |  | 8.11 |
|  | 35% | 5.50 | 2060 | 0.62 | 2925 | 264 |  | 7.82 |
|  | 40% | 7.65 | 2721 | 0.81 | 3337 | 367 |  | 7.42 |
|  | 45% | 10.46 | 3499 | 1.03 | 3765 | 501 |  | 6.98 |
|  | 50% | 14.07 | 4441 | 1.27 | 4196 | 675 |  | 6.58 |
|  | 55% | 18.39 | 5437 | 1.56 | 4627 | 881 |  | 6.17 |
|  | 60% | 23.88 | 6594 | 1.88 | 5054 | 1144 |  | 5.76 |
|  | 65% | 30.28 | 7796 | 2.21 | 5481 | 1451 |  | 5.37 |
|  | 70% | 37.89 | 9151 | 2.56 | 5907 | 1815 |  | 5.04 |
|  | 75% | 47.24 | 10678 | 3.01 | 6338 | 2263 |  | 4.72 |
|  | 80% | 58.51 | 12321 | 3.47 | 6769 | 2801 |  | 4.40 |
|  | 85% | 72.21 | 14094 | 3.96 | 7210 | 3456 |  | 4.08 |
|  | 90% | 88.66 | 16547 | 4.57 | 7696 | 4239 |  | 3.90 |
|  | 95% | 122.18 | 17393 | 5.63 | 8217 | 5832 |  | 2.98 |
|  | 100% | 121.67 | 17379 | 5.61 | 8203 | 5808 |  | 2.99 |

3.Outline Dimensions (mm)